Using the NewPing library with an Ultrasonic sensor

- First you will need to get the NewPing library as a zip file.  Place it on your desktop so you can find it easily.

- Wire up your sensor to the Arduino as such:

- Start  the Arduino IDE

- Add the NewPing library by going to the Sketch menu and selecting Add .ZIP Library…

- Navigate to the Desktop and select NewPing.zip

- After the NewPing Library is installed, Select it from the Sketch Include Library menu.

- When you select it, it will automatically add the following line to your sketch.

#include <NewPing.h>

- Here is a short demo program that you can run.

#include <NewPing.h>

#define TRIGGER_PIN  12           // Arduino pin tied to trigger pin on the ultrasonic sensor.

#define ECHO_PIN     11             // Arduino pin tied to echo pin on the ultrasonic sensor.

#define MAX_DISTANCE 200    // Maximum distance we want to ping for (in centimeters).

// Maximum sensor distance is rated at 400-500cm.

// NewPing setup of pins and maximum   distance.

NewPing   sonar(TRIGGER_PIN, ECHO_PIN, MAX_DISTANCE);

void setup() {

Serial.begin(115200);         // Open serial monitor at 115200 baud to see ping results.

}

void loop() {

// Wait 50ms between pings (about 20 pings/sec).

//29ms should be the shortest delay between pings.

delay(50);

Serial.print("Ping: ");

// Send ping, get distance in inches and print result (0 = outside set distance range)

Serial.print(sonar.ping_in());

Serial.println(" inches");

delay(1000);

}

- You can monitor the distance reported by opening the Serial Monitor and make sure your baud rate to 115200.